# Getting Started

# How to use SensiBLE 1.0 with ARDUINO IDE (Win10)

- Download and install the latest Arduino IDE.

- Launch Arduino IDE.
Click on File menu and then Preferences
The Preferences dialog will open, then add the following link to the Additional Boards Managers URLs field:

https://github.com/stm32duino/BoardManagerFiles/raw/master/STM32/package_stm_index.json

- Click Ok  
See below.

- Click on Tools menu and then “Boards > Boards Manager”
Next in the box type: stm32
and install all the boards that appears.
See below.

- Now select again: Tools > Boards
and choose the NUCLEO-64 board.

- Now select again: Tools > Board part number
and choose the NUCLEO L476RG

- Now select again: Tools 
and choose the next:

- U(S)ART support: “Enable (generic ‘Serial’)”

- USB support (if available): “None”

- USB speed (if available): “Low/Full Speed”

- Optimize: “Smallest (-Os default)”

- C Runtime Library: “Newlib Nano (default)”

- Upload method: “STMCubeProgrammer (SWD)”

- Now connect SED-PRO with SensiBLE to computer via USB,

select Tools > Board part number

and choose the available port:

- Download and install the STMCubeProgrammer

- Run the STMCubeProgrammer and select “Firmware upgrade” if available Serial number: “Old ST-LINK…”

- Press Open in update mode. If need, press twice.

- Press Upgrade and close all STMCubeProgrammer windows after correct upgrade.

- Now select again: Sketch > Include Library and choose the Manage Libraries…

- Next in the box type: SPBTLE_RF
and install next library.
See below

- Repeat #13 and next in the box type: HTS221
and install next library STM32duino HTS221.
See below

- Repeat #13 and next in the box type: LIS3MDL
and install next library STM32duino LIS3MDL.
See below

- Repeat #13 and next in the box type: LSM6DS3
and install next library STM32duino LSM6DS3.
See below

- Repeat #13 and next in the box type: LPS25HB
and install next library STM32duino LPS25HB.
See below

- Repeat #13 and next in the box type: VL53L0X
and install next library STM32duino VL53L0X.
See below

- Repeat #13 and next in the box type: VEML6075
and install next library SparkFun VEML6075 Arduino Library.
See below

- Download from https://github.com/SensiEDGE/SensiBLEDuino/blob/main/APDS9250.zip library for APDS9250.

- Install APDS9250 Library into Arduino IDE. Select again: Sketch > Include Library and choose the Add .ZIP Library…

- Select file APDS9250.zip and choose the Open

- Download from https://github.com/arduino-libraries/MadgwickAHRS library for MadgwickAHRS algorithm.

Select Code and Download ZIP

See below

- Install Madgwick Library into Arduino IDE. Select again: Sketch > Include Library and choose the Add .ZIP Library…

- Select file MadgwickAHRS-maste.zip and choose the Open

- Download SensiBLE.zip from https://github.com/SensiEDGE/SensiBLEDuino/blob/main/SensiBLE.zip, unpack to your work directory and to open SensiBLE.ino.

Press Upload button and open into your smartphone application “ST BLE Sensor”. Enjoy!